3) BLUETOOTH:

#include <SoftwareSerial.h>

SoftwareSerial Bluetooth(8, 9); // RX, TX

int LED = 12; // the on-board LED

int Data; // the data received

void setup() {

Bluetooth.begin(9600);

Serial.begin(9600);

Serial.println("Waiting for command...");

Bluetooth.println("Send 1 to turn on the LED. Send 0 to turn Off");

pinMode(LED, OUTPUT);

}

void loop() {

if (Bluetooth.available()) { // wait for data received

Data = Bluetooth.read();

if (Data == '1') {

digitalWrite(LED, HIGH);

Serial.println("LED On!");

Bluetooth.println("LED On!");

}

else if (Data == '0') {

digitalWrite(LED, LOW);

Serial.println("LED Off!");

Bluetooth.println("LED Off ! ");

}

else {

;

}

}

delay(1000);

}

================================================================================

4 i) RFID Read:

#include <SPI.h>

#include <MFRC522.h>

#define RST_PIN 9

#define SS_PIN 10

MFRC522 mfrc522(SS_PIN, RST_PIN);

void setup() {

Serial.begin(9600);

SPI.begin();

mfrc522.PCD_Init();

Serial.println(F("Read personal data"));

}

void loop() {

MFRC522::MIFARE_Key key;

for (byte i = 0; i < 6; i++)

key.keyByte[i] = 0xFF;

byte block;

byte len;

MFRC522::StatusCode status;

if (!mfrc522.PICC_IsNewCardPresent()) {

return;

}

if (!mfrc522.PICC_ReadCardSerial()) {

return;

}

Serial.println(F("**Card Detected:**"));

mfrc522.PICC_DumpDetailsToSerial(&(mfrc522.uid));

Serial.print(F("Name: "));

block = 4;

len = 18;

byte buffer2[18];

block = 1;

status = mfrc522.PCD_Authenticate(

MFRC522::PICC_CMD_MF_AUTH_KEY_A,

1,

&key,

&(mfrc522.uid)

);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Authentication failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

status = mfrc522.MIFARE_Read(block, buffer2, &len);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("Reading failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

for (uint8_t i = 0; i < 16; i++) {

Serial.write(buffer2[i]);

}

Serial.println(F("\n**End Reading**\n"));

delay(1000); //change value if you want to read cards faster

mfrc522.PICC_HaltA();

mfrc522.PCD_StopCrypto1();

}

============

ii) RFID WRITE:

#include <SPI.h>

#include <MFRC522.h>

#define RST_PIN 9

#define SS_PIN 10

MFRC522 mfrc522(SS_PIN, RST_PIN);

void setup() {

Serial.begin(9600);

SPI.begin();

mfrc522.PCD_Init();

Serial.println(F("Write personal data on a MIFARE PICC "));

}

void loop() {

MFRC522::MIFARE_Key key;

for (byte i = 0; i < 6; i++)

key.keyByte[i] = 0xFF;

if (!mfrc522.PICC_IsNewCardPresent()) {

return;

}

if (!mfrc522.PICC_ReadCardSerial()) {

return;

}

Serial.print(F("Card UID:"));

for (byte i = 0; i < mfrc522.uid.size; i++) {

Serial.print(mfrc522.uid.uidByte[i] < 0x10 ? " 0" : " ");

Serial.print(mfrc522.uid.uidByte[i], HEX);

}

Serial.print(F(" PICC type: "));

MFRC522::PICC_Type piccType = mfrc522.PICC_GetType(mfrc522.uid.sak);

Serial.println(mfrc522.PICC_GetTypeName(piccType));

byte buffer[34];

byte block;

MFRC522::StatusCode status;

byte len;

Serial.setTimeout(20000L);

// Ask personal data: First name

Serial.println(F("Type First name, ending with #"));

len = Serial.readBytesUntil('#', (char *) buffer, 20);

for (byte i = len; i < 20; i++)

buffer[i] = ' ';

block = 1;

status = mfrc522.PCD_Authenticate(

MFRC522::PICC_CMD_MF_AUTH_KEY_A,

block,

&key,

&(mfrc522.uid)

);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

status = mfrc522.MIFARE_Write(block, buffer, 16);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("MIFARE_Write() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

else Serial.println(F("MIFARE_Write() success: "));

block = 2;

status = mfrc522.PCD_Authenticate(

MFRC522::PICC_CMD_MF_AUTH_KEY_A,

block,

&key,

&(mfrc522.uid)

);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("PCD_Authenticate() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

status = mfrc522.MIFARE_Write(block, &buffer[16], 16);

if (status != MFRC522::STATUS_OK) {

Serial.print(F("MIFARE_Write() failed: "));

Serial.println(mfrc522.GetStatusCodeName(status));

return;

}

else Serial.println(F("MIFARE_Write() success: "));

Serial.println(" ");

mfrc522.PICC_HaltA();

mfrc522.PCD_StopCrypto1();

}

======================================================================================

5)  Temperature and Humidity:

#include <DHT.h>

#define DHTPIN 8

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

Serial.println(F("DHT test!"));

dht.begin();

}

void loop() {

delay(2000);

float h = dht.readHumidity();

float t = dht.readTemperature();

if (isnan(h) || isnan(t)) {

Serial.println(F("Failed to read from DHT sensor!"));

return;

}

Serial.print(F(" Humidity: "));

Serial.print(h);

Serial.print(F("% Temperature: "));

Serial.print(t);

}

=========================================================================

6) IR Sensor:

#include <ESP8266WiFi.h>

String apiKey = "PFFHC1B2IRN0GGQ0";

const char *ssid = "Veeresh";

const char *pass = "veeru1987";

const char* server = "api.thingspeak.com";

#define IRpin D4

WiFiClient client;

int value;

void setup() {

Serial.begin(115200);

pinMode(IRpin, INPUT);

delay(1000);

Serial.println("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED) {

delay(2000);

Serial.print(".");

}

Serial.println("   ");

Serial.println("WiFi connected");

}

void loop() {

value = digitalRead(IRpin);

Serial.println(value);

if (value == 0) {

Serial.print("object detected");

}

else {

Serial.print("no object detected");

}

if (client.connect(server, 80)) {

String postStr = apiKey;

postStr += "&field1=";

postStr += String(value);

postStr += "\r\n\r\n";

client.print("POST /update HTTP/1.1\n");

client.print("Host: api.thingspeak.com\n");

client.print("Connection: close\n");

client.print("X-THINGSPEAKAPIKEY: " + apiKey + "\n");

client.print("Content-Type: application/x-www-form-urlencoded\n");

client.print("Content-Length: ");

client.print(postStr.length());

client.print("\n\n");

client.print(postStr);

client.stop();

Serial.println("Waiting...");

delay(1000);

}

}

==================================================================================

7) Upload Temperature and Humidity Cloud:

#include <DHT.h>

#include <ESP8266WiFi.h>

String apiKey = "E5E5FA7AF200H6I6";  //

const char *ssid = "Anuja";

const char *pass = "Anuja123";

const char* server = "api.thingspeak.com";

#define DHTPIN D3

DHT dht(DHTPIN, DHT11);

WiFiClient client;

void setup() {

Serial.begin(115200);

delay(1000);

dht.begin();

Serial.println("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED) {

delay(2000);

Serial.print(".");

}

Serial.println("");

Serial.println("WiFi connected");

}

void loop() {

float h = dht.readHumidity();

float t = dht.readTemperature();

if (isnan(h) || isnan(t)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

if (client.connect(server, 80)) {   //   "184.106.153.149" or api.thingspeak.com

String postStr = apiKey;

postStr += "&field1=";

postStr += String(t);

postStr += "&field2=";

postStr += String(h);

postStr += "\r\n\r\n";

client.print("POST /update HTTP/1.1\n");

client.print("Host: api.thingspeak.com\n");

client.print("Connection: close\n");

client.print("X-THINGSPEAKAPIKEY: " + apiKey + "\n");

client.print("Content-Type: application/x-www-form-urlencoded\n");

client.print("Content-Length: ");

client.print(postStr.length());

client.print("\n\n");

client.print(postStr);

Serial.print("Temperature: ");

Serial.print(t);

Serial.print(" degrees Celcius, Humidity: ");

Serial.print(h);

Serial.println("%. Send to Thingspeak.");

}

client.stop();

Serial.println("Waiting...");

delay(1000);

}

==================================================================

8) Retrieve Temperature and Humidity Cloud:

#include <DHT.h>

#include <ESP8266WiFi.h>

String apiKey = "UYK6QJGVZPSG5LF2";   //read

const char *ssid = "Anuja";

const char *pass = "Anuja123";

const char* server = "api.thingspeak.com";

#define DHTPIN D3

DHT dht(DHTPIN, DHT11);

WiFiClient client;

void setup() {

Serial.begin(115200);

delay(1000);

dht.begin();

Serial.println("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("");

Serial.println("WiFi connected");

}

void loop() {

float h = dht.readHumidity();

float t = dht.readTemperature();

if (isnan(h) || isnan(t)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

if (client.connect(server, 80)) {   //   "184.106.153.149" or api.thingspeak.com

String postStr = apiKey;

postStr += "&field1=";

postStr += String(h);

postStr += "&field2=";

postStr += String(t);

postStr += "\r\n\r\n";

client.print("X-THINGSPEAKAPIKEY: " + apiKey + "\n");

client.print("Content-Length: ");

client.print(postStr.length());

client.print("\n\n");

client.print(postStr);

Serial.print("Temperature: ");

Serial.print(t);

Serial.print(" degrees Celcius, Humidity: ");

Serial.print(h);

Serial.println("%. Send to Thingspeak.");

}

client.stop();

Serial.println("Waiting...");

delay(1000);

}

========================================================================================

9) TCP:

#include "ESP8266WiFi.h"

#include "DHT.h"

const char* ssid = "Anuja";

const char* password = "Anuja123";

WiFiServer wifiServer(8080);

DHT dht(D3, DHT11);

void setup() {

Serial.begin(115200);

delay(1000);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println("Connecting..");

}

Serial.print("Connected to WiFi. IP:");

Serial.println(WiFi.localIP());

wifiServer.begin();

dht.begin();

}

void loop() {

WiFiClient client = wifiServer.available();

if (client) {

while (client.connected()) {

while (client.available() > 0) {

float t = dht.readTemperature();

float h = dht.readHumidity();

client.println("humidity :");

client.println("temperature :");

client.println(h);

Serial.println(h);

client.println(t);

Serial.println(t);

delay(2000);

}

}

client.stop();

Serial.println("Client disconnected");

}

}

==========================================================================

10) UDP

#include <ESP8266WiFi.h>

#include <WiFiUdp.h>

#include <DHT.h>

const char* ssid = "Anuja";

const char* password = "Anuja123";

const char* udpAddress = "192.168.117.39";

const int udpPort = 8080;

#define DHTPIN D3

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

WiFiUDP udp;

void setup() {

Serial.begin(115200);

Serial.println();

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.print("Connecting");

}

Serial.println();

Serial.print("Connected to WiFi.IP:");

dht.begin();

}

void loop() {

delay(10000);

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

if (isnan(temperature) || isnan(humidity)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.print(" °C\tHumidity: ");

Serial.print(humidity);

Serial.println(" %");

Serial.println("Sending data over UDP...");

udp.beginPacket(udpAddress, udpPort);

udp.print("Temperature: ");

udp.print(temperature);

udp.print(" °C, Humidity: ");

udp.print(humidity);

udp.println(" %");

udp.endPacket();

Serial.println("Data sent over UDP.");

}